Task A: Clean the Stream (Code & Inspection)

Inspection Statistics

- Expected readings: 288 (every 5 minutes for 24 hours)

- Arrived readings: 254 (34 dropped / lost packets, ~12%)

- Impossible readings: 3 readings fall outside the sensible indoor range of 5∘C to 40∘C (-40.0∘C, 85.0∘C, and 0.0∘C).

Task B: Evaluate (MAE Comparison)

MAE Results Table

| Cleaning Stage | Mean Absolute Error (MAE) | Number of Readings Compared |
| --- | --- | --- |
| Raw Data | 2.64∘C | 254 |
| After Edge Cleaning | 2.81∘C | 251 (3 impossible values dropped) |
| After Cloud Cleaning | 0.21∘C | 288 (reindexed to full grid & interpolated) |

Explanation

The raw data has a moderate MAE due to extreme glitches and the stuck sensor block. After edge cleaning, the MAE temporarily increases because removing the extreme outliers (like 85∘C and -40∘C) leaves gaps rather than correcting the values, altering the denominator/count. Finally, cloud cleaning drops the error dramatically because it re-establishes the complete 288-reading time grid, neutralizes the stuck sensor, and fills gaps via interpolation, bringing the stream very close to the truth.

Task C: Edge or Cloud?

| Cleaning Step | Edge or Cloud? | Reason (Real Constraint) |
| --- | --- | --- |
| Range Check (5–40°C) | Edge | Battery / Bandwidth: Filtering impossible values locally prevents transmitting corrupt data over the network, saving radio transmission power. |
| Stuck Sensor Detection | Cloud | Needing History / Memory: Detecting a stuck sensor requires analyzing a rolling window of multiple past readings over time, exceeding the memory and state storage of a constrained edge sensor. |
| Gap Filling / Interpolation | Cloud | Computation / Memory: Reindexing to a full timestamp grid and performing temporal interpolation requires global dataset visibility and heavier compute resources. |

Task D: Features & Selection

1. Feature Engineering

- hour_of_day: The hour component of the timestamp (capturing diurnal temperature patterns).

- rolling_avg_1h: A 1-hour rolling average (12 periods) of temperature (capturing short-term moving trends).

- temp_change_5min: The difference between the current reading and the reading 5 minutes prior (capturing immediate rate of change).

2. Target Creation & Correlation Check

- Target: will_be_warmer_1h (Binary: 1 if temperature 60 minutes ahead >21∘C, else 0).

- Correlation check: rolling_avg_1h and hour_of_day show strong positive/cyclical correlation with the target. temp_change_5min shows weak correlation.

3. Keep / Drop Decisions

- Keep rolling_avg_1h: Strongly correlates with future temperature as it smooths out noise and captures the local heating/cooling trend.

- Keep hour_of_day: Highly predictive because building temperature is heavily driven by scheduled HVAC cycles and time-of-day solar gain.

- Keep temp_change_5min: Retained as a momentum indicator for sudden thermal shifts.

- Drop raw_timestamp_numeric (or secondary derivative): Dropped because redundant time representations do not add unique predictive value beyond hour_of_day.

Task E: Stretch Challenge (Option 3: Privacy)

Scenario Choice: Privacy (What to aggregate or remove before data leaves the building, and what is lost).

Temperature sensors in a meeting room might seem completely non-sensitive, but rapid, localized spikes or drops in temperature can reveal occupancy patterns—such as exactly when people enter, how many people are present, and when meetings start and end.

Before the data leaves the building, high-frequency fluctuations should be mitigated by applying temporal downsampling (e.g., transmitting 30-minute averages instead of 5-minute raw telemetry) or adding differential privacy noise. What is lost by doing this is high-resolution responsiveness: real-time anomaly detection, immediate thermal feedback control for HVAC systems, and fine-grained occupancy analytics become impossible. However, the trade-off successfully protects occupant privacy by obscuring individual meeting schedules and presence patterns.